TRAFFIC CLASSIFICATION SYSTEM

Mininet + POX SDN Controller

Operating Systems Project  |  Network Traffic Analysis

# 1. Project Overview

The Traffic Classification System is an SDN project using Mininet and the POX OpenFlow controller to capture, inspect, and classify live network traffic by protocol type in real time. Every packet flowing through the virtual network is intercepted by the controller, analyzed at the IP and transport layers, and displayed in a color-coded terminal dashboard.

Core Objective: Classify every network packet as TCP, UDP, ICMP, or OTHER — maintaining live statistics (packet count, bytes transferred, percentage distribution) and displaying them in the terminal.

# 2. System Architecture

The system uses the SDN model where the control plane (POX) is decoupled from the data plane (Open vSwitch in Mininet).

### 2.1  Network Topology

| Node | Details |
| --- | --- |
| h1, h2, h3, h4 | Virtual hosts with IPs 10.0.0.1 – 10.0.0.4/24 |
| s1 (OVS Switch) | Open vSwitch in OpenFlow mode, connected to POX |
| POX Controller | Listens on 127.0.0.1:6633, classifies all packets |
| Links | 10 Mbps bandwidth, 5ms delay per link |

### 2.2  Data Flow

- Host sends a packet (ping, iperf, etc.) inside Mininet.

- Open vSwitch intercepts it and sends it to POX via an OpenFlow PacketIn event.

- POX traffic_classifier.py receives the raw packet data.

- Classifier inspects Ethernet → IP header → Transport layer.

- Statistics updated (count + bytes) and terminal display refreshed.

- Controller sends packet back out via PacketOut flood so traffic still flows.

# 3. Classification Logic

| Protocol | Packets | Bytes | Distribution | Traffic Type |
| --- | --- | --- | --- | --- |
| TCP | Counted | Tracked | Bar chart % | iperf streams, SSH, web |
| UDP | Counted | Tracked | Bar chart % | iperf -u, DNS, streaming |
| ICMP | Counted | Tracked | Bar chart % | ping, network probes |
| OTHER | Counted | Tracked | Bar chart % | ARP, non-IP frames |

### 3.1  Detection Order (per packet)

- Find IPv4 layer → extract source IP and destination IP

- If TCP sublayer found → classify TCP, capture src/dst ports

- Else if UDP sublayer found → classify UDP, capture src/dst ports

- Else if ICMP sublayer found → classify ICMP

- No IPv4 layer (ARP etc.) → classify OTHER

# 4. Project Files

| File | Purpose |
| --- | --- |
| traffic_classifier.py | POX controller — core engine. Handles PacketIn events, classifies protocols, maintains stats, renders terminal display. |
| topology.py | Mininet topology. Creates 4 hosts + 1 switch, connects to POX. Includes run_demo and show_stats CLI commands. |
| setup.sh | Automated setup. Installs POX if needed, copies controller, prints startup instructions. |

# 5. Setup & Run Instructions

### Step 1 — Install POX

git clone https://github.com/noxrepo/pox.git ~/pox

### Step 2 — Copy controller into POX

cp ~/Downloads/os_orange2/traffic_classifier.py ~/pox/pox/misc/

### Step 3 — Full cleanup (run before every session)

sudo pkill -f pox.py && sudo mn -c && sleep 3

### Step 4 — Terminal 1: Start POX

cd ~/pox && python3 pox.py misc.traffic_classifier

### Step 5 — Terminal 2: Start Mininet

sudo python3 ~/Downloads/os_orange2/topology.py

### Step 6 — Generate Traffic in Mininet CLI

| Command | Effect |
| --- | --- |
| h1 ping -c 5 h2 | Sends 5 ICMP packets from h1 to h2 |
| h1 iperf -s & | Starts TCP iperf server on h1 |
| h2 iperf -c 10.0.0.1 | Runs TCP iperf client, connects to h1 |
| h3 iperf -u -s & | Starts UDP iperf server on h3 |
| h4 iperf -u -c 10.0.0.3 | Runs UDP iperf client, connects to h3 |
| run_demo | Auto-generates mixed TCP + UDP + ICMP traffic |
| pingall | Tests all-pairs ICMP connectivity |

# 6. Terminal Output

The POX terminal clears and redraws every time a packet is classified. It shows four sections:

| Section | Content |
| --- | --- |
| Header Bar | Uptime (seconds), total packet count, current timestamp |
| Protocol Table | Per-protocol: packet count, bytes, percentage share, ASCII bar chart |
| Last 10 Packets | Rolling log: time, protocol, src/dst IP, src/dst port, size in bytes |
| Latest Packet | Highlighted summary of the most recently classified packet |

# 7. Troubleshooting

| Error / Symptom | Fix |
| --- | --- |
| Address already in use (port 6633) | sudo pkill -f pox.py && sleep 3, then restart POX |
| RTNETLINK: File exists | sudo mn -c, then restart Mininet |
| Module not found: misc.traffic_classifier | cp traffic_classifier.py ~/pox/pox/misc/ |
| Packets not updating in Terminal 1 | In Mininet: sh ovs-ofctl del-flows s1, then ping again |
| No stats file found (show_stats) | Normal — stats show in Terminal 1 (POX), not written to file |

# 8. Requirements

| Tool | Install Command |
| --- | --- |
| Mininet | sudo apt install mininet |
| Open vSwitch | sudo apt install openvswitch-switch |
| POX Controller | git clone https://github.com/noxrepo/pox.git ~/pox |
| Python 3 | Pre-installed on Ubuntu (best with Python 3.6–3.9) |
| iperf | sudo apt install iperf |

Note: Wireshark is NOT required. POX captures packets directly via OpenFlow PacketIn events and displays everything in the terminal.